Cell-Devs Model of Hydraulic Fracturing with Lopez

Bowen Yang             Gabriel Wainer

Faculty of Engineering

University of Ottawa

161 Louis Pasteur,

Ottawa, ON K1N 6N5

byang078@uottawa.ca        gwainer@sce.carleton.ca

Abstract: Hydraulic Fracturing is a well-stimulation technique, nowadays widely used in wells for exploiting gas. Building a model for hydraulic fracturing will help the study for learning the pattern and phenomenon of rocks cracked by fluid with increased pressure. The model is based on Lopez which is the one of the version of CD++ and allows multiple state values and ports in a single cell. In this paper, the process of how to build a model with Lopez and RISE api, and the detail of the hydraulic fracturing model will be shown.

# Introduction

- The principle of hydraulic fracturing is injecting high-pressured fluid (usually water suspended with sand and mixed with other chemicals) into a wellbore to crack the rock formulation then releases the gas and oil which will flow more freely. After the fluid completes their work, they will be pulled out and cracks will be filled by other chemicals. The main purpose of this project is to simulate the process of how fluid crack the rock formulation step by step.

- Figure 1.1: Hydrulic fracturing

- The whole process can be subdivide into continuous pressure increased - crack and release the pressure activities. Each crack happens independently, so that the whole process can be expressed by cellular automata.

- To build this model, we may need to store several state variables into a cell. With previous version of CD++, when we want to store multiple variables, we can only set up extra dimensions. This way can solve such problems correctly, but it may be complicated especially when there are already more than 3 dimensions in the model.

- The new version of CD++, Lopez, provide us a more convenient way. New state variables and new neighbor port can be added into the model.Actually, Lopez still implements such functions by add new dimensions in to the model. However, it provide us a easier way.

# Background

- The improvement of Lopez is the additional state variables and ports with each cells. And the details of how to use them are shown below:

## 2.1 Declaration

First, all state variables and neighbor ports should be declared.

- State variables is declared after the keyword StateVariables as a list and separated by spaces. Then they should be initialized by using keyword StateValues which the list of state variable’s initial value should be listed after it separated by spaces, or using a file to initialize different value of different state variables that the file name should follow the keyword InitialVariablesValue.

- As for neighbor ports, the declaration is quite similar as state variables that all the neighbor ports’ name should be listed and follow the keyword NeighborPorts.

## 2.2 Rules

There are some modifications of the rules in Lopez, the format of rules is shown below:

{port_assignations} [ {assignations} ] <delay> {conditions}

In the previous version of CD++, the first part of a rule is the formula that represents the way how the values of every cells are modified in each iteration.

There are multiple state variables in Lopez, so that there could be multiple formulas represents the modifications of each variables and ports.

- The formulas of variables and ports should be separated into two parts and be enclosed by brackets separately. Format of formula is shown below, each formulas should followed by a semi-colon for separating.

- <state variable/neighbor port> := <Expression>;

- To reading values from other cells, ore use the values stored in the own cell, a cell can read data from neighbor’s neighbor port or from state its own variables. There should be a “~” before all the neighbor ports and a “$” before all state variables when they are part of a expression. What’s more, to read data from others’ neighbor ports, the position of the cell should be displayed in front of the “~”.

# 3.Model Definition

## 3.1 Problem Description

The model is based on a 3-dimensional coordinate system, simulating the process of hydraulic fracturing. There is already a well bore exists which is the source of the pressure of liquid. The pressure will be increased continuously and transferred to other places that have been filled with liquid. When the pressure exceeds the nearby rock’s limit, the rock will be cracked and turns to be fracturing. What’s more, cracked rock  at that position will be replaced by liquid after that.

The maximum support pressure of rock is related to 2 parameters. One of them is the depth of the rock, the deeper the rock is, the more pressure it can support.

Another parameter is degree of how much the liquid compress the rock. When fracturing takes place somewhere, parts of rock at that position will be compressed into nearby rock formulation of which the density and the maximum support pressure will be promoted.

Figure 3.1 How liquid compress the rock.

The blue part represents liquid, the brown parts represent rock, and the red arrows represent the pressure of liquid

However, what should be concerned is that that tip of the fault will not be effect by the compression.

## 3.2 Conceptual Outline

Each cell of the model will contain 3 state value.

The first one is the state of such position, called cstate. It can take 5 possible values, and the relationship with the value and the statue is shown below.

Table 1. cstate value relationship

| Possible value | State represented |
| --- | --- |
| 0 | Rock |
| 1 | Fracturing |
| 2 | Fault tip |
| 3 | Liquid |
| 4 | Well bore |

At the beginning , all cells except one cell which represents well bore are all represent rock. When the rock somewhere can’t support the liquid pressure anymore, it will be fracturing and then become fault tip. A fault tip will turn to full liquid when there is another rock nearby cracked and become fracturing.

The second state value is cpress which the pressure of liquid. When the value cstate is 0 which means the cell represents the rock, cpress will keep 0 and make no sense.

When the cell no more represent the rock, which means such position has been filled with liquid. At that time, the cpress will start increasing,

The pressure of one position is related by its nearby position. liquid will move from the place with high pressure to the place with low pressure which will balance the pressure automatically. So the pressure of a cell that doesn’t represent rock or well bore will be the average of the pressure of all its non-rock neighbor and itself.

The well bore is the only different cell at the beginning. And another specification of it is that the pressure there is not only the average with neighbors, but it will also increased constantly in each iteration,

The third value compress is the density of the rock. Opposite with the liquid pressure, the density makes no sense when a cell is not representing rock. When rock get fracturing, compress of other rock nearby will be increased.

This value will influence the maximum supported pressure.

Each cell will contain 3 neighbor port as well.They are cstateport, press, compressport which is the port of 3 state variables for neighbors to read value.

In each iteration, with the different state of cells, there are different moves will be active.

Table 2: Possible moves with different state

| cstate Value | Possible moves |
| --- | --- |
| 0 | Check if it will be cracked and increase compress if neighbor is cracked |
| 1 | Turn the state to fault tip and get average of pressure |
| 2 | Turn the state to liquid if neighbor is cracked and get average of pressure |
| 3 | Get average of pressure |
| 4 | Increase the pressure constantly |

## 3.3 Fomral Definition

HF=<X , Y , I , S ,θ, N, D , δext , δint ,τ, λ, D>

X={}

Y={}

I = <η, μ, Px, Py>

η = 27

μ = 0

Px = {all in neighborhood}

Py = {all in neighborhood}

θ={

cstate=0:rock

cstate=1:fracturing

cstate=2:fault tip

cstate=3:liquid

cstate=4:well bore

}

N={}

D=transport delay 100ms

δext={}

δint=As table 2 shows. Detailed information is shown below:

### Rock rule:

First, check the neighbors that don’t represent rock. If one of neighbors’ liquid pressure exceeds the maximum supported pressure, this cell will be cracked.

The maximum supported pressure is calculated as this:

pmax=pd∗(1+not fault tip*density)

wherepd represent the part of maximum supported pressure related to the depth, and the formula of pd is shown below:

pd=2σ−pf+T0

and

Table 3: Properties of pd

| Symbol | Description | value |
| --- | --- | --- |
| p | basic density | 22008 |
| T0 | rock tensile strength | 15000000 |
| g | gravity acceleration | 9.8 |
| gP | pore fluid pressure gradient | 10500 |
| pf | pore fluid prssure | Depth*gP |
| σ | stress | Depth*g*p |

In this model, for making the result more realistic, the parameter Depth should be set as (cellpos(0)+constant depth).

After simplify, the formula of pmax is shown below:

pmax=(cellpos(0)∗DepthCoefficient+constant_depth)*299800+150000)∗(1+not_fault_tip*density)	where the default depth coefficient is 1.0, and the default constant depth is 3000.

If exceeding the maximum supported pressure, the rock will be cracked and get fracturing.

If rock won’t be fracturing after check the maximum supported pressure at that time, it will check the neighbors to find if there is a neighbor is cracked currently.

if there is a cracked neighbor, add the density by the cracked neighbor’s multiply a coefficient. In this project, I set the coefficient as 0.5;

The formula will be:

### density=density+neighbor|cstate=1density∗                                    CompressionCoefficient

where the default compression coefficient is 0.5.

### Fracturing rule:

Fracturing is a intermediate state between rock and fault tip. This type is used to show a message to its neighbor that one of the neighbor has been cracked.

Once a piece of rock has been cracked into fracturing. It will turn to the fault tip state in next iteration.

It will still set the pressure to the average of neighbors’ liquid pressure.

### Fault tip rule:

There is only one thing between a cell of

fault tip and a cell of liquid that a rock cell with liquid neighbor will get density bonus but this is not applied with fault tip neighbor.

A fault tip cell will turn to liquid cell when there is a fracturing neighbor occurs.

### Liquid rule:

In each iteration, a liquid cell will set its pressure to the average liquid pressure of all neighbor that don’t represent rock.

The formula is shown below:

pressure=cstate≠0pressurecstate≠0

### Well bore rule:

This is the special cell that will increase the pressure by a constant value in every iterations.

## 3.4 Implement

Neighbors definition.

neighbors:startum(-1,-1,-1) startum(-1,0,-1) startum(-1,1,-1) startum(0,-1,-1) ......

startum(1,0,1)  startum(1,1,1)

Ports, state variables and local transition definition.

localtransition : hfrule

StateVariables : cstate

StateVariables : compress

StateVariables : cpress

neighborports : press

neighborports : cstateport

neighborports : compressport

Rules for well bore.

rule : {~press := $cpress; ~compressport := 0; ~cstateport := 3; }

{ $cpress := $cpress + 50000000; }

100

{ $cstate =4 }

Rules for liquid cell

rule:{~press := $cpress;~cstateport:= $cstate; } { $cpress := #macro(meanofpress); } 100

{$cstate = 3 or $cstate = 2}

Rules for fault tip cell

rule:{~press:=$cpress; ~cstateport := $cstate; } { $cstate := 3; $cpress := #macro(meanofpress); } 100

{($cstate=2)and(stateCount(1,~cstateport)>0)}

Rules for fractured cell

rule : { ~press := $cpress; ~cstateport := $cstate; ~compressport:=$compress;}

{$cstate:=2; $cpress := #macro(meanofpress);  $compress :=0; } 100

{($cstate = 1)}

Rules for rock cell

rule:{~press:= $cpress; ~cstateport := $cstate; } { $cstate:= 1; $cpress := 1; } 100

{($cstate=0)and((-1,-1,-1)~press> #macro(threshold))}

...

rule:{~press:= $cpress; ~cstateport := $cstate; } { $cstate:= 1; $cpress := 1; } 100

{($cstate=0)and((1,1,1)~press> #macro(threshold) ) }

rule:{~press := $cpress; ~compressport := $compress;}

{ $compress := $compress + #macro(compressrock); $cpress := 0;} 100 {t}

Macro#1: Caculate the average of the liquid pressure of all neighbors.

(if((-1,-1,-1)~press!=?,(-1,-1,-1) ~press, 0) +

if((-1,0,-1)~press != ?, (-1,0,-1) ~press, 0) +

....

if( (1,1,1)~press != ?,(1,1,1) ~press, 0)) /

(27-stateCount(0,~cstateport)-

stateCount(?,~cstateport))

Macro#2: Caculate the maximum supported pressure of a rock cell.

((cellPos(0)*100+1600)*299800+150000) *

(1+if(stateCount(2,~cstateport)=1,0,1)*$compress )

Macro#3: Increase the density when there is a cracked neighbor.

(

if((-1,-1,-1)~cstateport=1,(-1,-1,-1)~compressport*0.7,0)+

if((-1,0,-1)~cstateport=1,(-1,0,-1) ~compressport*0.7,0)+

...

if( (1,1,1)~cstateport = 1,(1,1,1) ~compressport*0.7,0)

)

# 4.Simulation Result

I have wrote another program to display the section of whole cell space.

The figures below are all section of the space.And shapes with different lightness represent cell with different states.

For making the result more realistic, the initial state value compress of every cells are generated randomly from 0 to 5.

When the simulation begins, the well bore(which is on (7,0,7)) is the only cell that doesn’t represent rock. (Figure 4.1)

Figure 4.1: time=500 z=7

Figure 4.2: time=600. z=(6), (7), (8)

As Figure 4.2 shows, with the increased liquid pressure from the well bore, several rock cell has been cracked and turned to fracturing state.

Figure 4.3: time=700ms z=7

With the new rock cell turns to be fracturing, the cells that cracked in previous iteration has turned to be fault tip and liquid.

Figure 4.4:time=(900), (1000) z=7

In figure 4.4, the rock cell in (7,1,7) keeps not being fractured when most of its neighbors have been fractured. This is because of the effect of liquid compress.

Figure 4.5: time=3500ms (z=7), (y=0)

After several iterations, more rock cells are already cracked, if the simulation goes on,all the rock will be cracked.

In other simulations, the coefficient of the compression ,the coefficient of depth and the constant depth are modified separately. The results are shown below:

Figure 4.6: time=5000ms (z=7), (y=0)

Figure 4.6 shows the result after the coefficient of compression is modified larger to 0.85. It’s easy to see that with the larger number of iterations, the number of cracked rock cell is less than the result shown in figure 4.5.

A larger coefficient of compression means that when a rock cell is cracked, its neighbor will turn to be more harder. So the rock cells that don’t contact with fault tips will be more difficult to be fractured.

Figure 4.7 time=3000ms z=7

Figure 4.7 shows the result after modifying the coefficient of depth to 20 and set constant depth to 1600. Which will strength the influence to the hardness of rock of the depth, at the same time won’t change the result at the beginning.

From the result, it shows that there are more cells in the area of (x<7) that are cracked than cells in the area of (x>7).

- Figure 5.1: t=(10000),(28000)ms z=7

This is the result with a long simulation. It can be seen that the space is not big enough over long time.

# 5.Discussion

- Compared with the example model, this model runs in 3-Dimensional space, and can get a more realistic result with change of depth.

- New version of CD++ also helps a lot with building this model.

- There are still a defect in this model.

- The first one is when a liquid cell with the pressure exceeds the maximum supported pressure of its several neighbors. In real world, what will happen is that only one of its neighbor will be cracked. But in this model, all neighbors that can’t support such pressure will be cracked.

- To solve this problem, a potential method is add another state value that select a direction randomly when there are multiple options. And a rock cell will be cracked only when both pressure and the direction condition are fit.

- I failed to achieve this method because of there is too much conditions need to be determined when there are 27 neighbors with a cell. I need to find a approach to determine such condition in a clear way.

# Conclusion

- This project is about building and upgrade a model simulating the process of hydraulic fracturing with new version CD++ based on a sample model and paper. The model completes most of requirements but still leaves some room for improvement because of the time constraint. I will continue working on this model in the future.

# References

[1]Hubbert M K, Willis D G. Mechanics of hydraulic fracturing[J]. 1972.

[2]Christopher Burt. Simulating Hydraulic Fracturing Using Cell-DEVS. Carelton University.

[3]A López, G. Wainer. 2003. Extending CD++ Specification Language for Cell-DEVS Model

Definition.

[4]K. Al-Zoubi and G. Wainer. 2010. RESTful Interoperability Simulation Environment (RISE)

Middleware Manual.