SYSC-5104 
Methodologies for Discrete-Event Modelling and Simulation (Fall 2016)

#### Assignment1: HomNet Simulators

# Victor Guimaraes da Silva  101022659

Carleton University

# Homogeneous Networks

Due to the increasingly demand for mobile data, operators have been using different techniques to deal with this challenge. One of the factors that influence the overall network performance is the efficient use of the radio spectrum. One approach suggested by the 3rd Generation Partnership Project (3GPP) is the use of Heterogeneous Networks (HetNets) which assist on the reuse of the radio resources. HetNets are composed of various kinds of wireless access nodes with varied capacities. In the LTE-Advanced and 5G cellular networks, HetNets are a combination of macro-cells (MeNBs) and low-power nodes, for instance, pico-cells (PeNB), femto-cells, Remote Radio Heads (RRHs), and relay nodes.

Even though HetNets are able to improve the cell performance, it is still necessary to improve the control algorithms regarding the handover (HO) process because increasing the number of eNBs on a determined area can increase the amount of handovers and control messages which can bring significant amount of overhead which worsen the cell performance.

Initially, the goal of this project is to first show a quantitative analysis of the handover process on HomNets, specifically the amount of handovers. Later on, the HetNet case will be also showed, however for this, only the Homogeneous case will presented.

In short, the conventional handover process is made to avoid any service interruption when a user is going out of a determined cell coverage. If the receiving power is not enough, the User Equipment will connect to another eNB.

The User Equipment (UE) measures the Serving eNB`s receiving power (named RSRP in the 3GPP standard) every 40 ms. The UE also measures the second highest RSRP (Target eNB, or TeNB), periodically.

If the Serving eNB (SeNB) RSRP is lower than a threshold (in comparison to the TeNB receiving power, named Handover criteria) the UE takes five RSRP measures (one every 40 ms) and averages out the measurements. If the averaged RSRP meets the Handover criteria the UE sets a time-to-trigger (TTT) of 160 ms, which makes the UE wait that amount of time until measuring the RSRP again. After waiting and measuring again, the UE sends (trigger) a Measurement Report (MR) to SeNB. After receiving the MR, SeNB checks the MR and takes HO decision. The SeNB will send HO request to TeNB and TeNB sends HO request acknowledgement (ACK) to SeNB.  The SeNB sends RRC Connection Reconfiguration message (HO Command) to UE. Finally, UE sends RRC Connection Reconfiguration Complete message to TeNB (Connect to the TeNB).

It is important to notice that the process previously described happens whether the UE is moving or not. Therefore, the UE will have a specified velocity and position. The UE position is calculated before the RSRP. In order to simplify the simulator, the UE has a uniform linear motion with constant velocity. The UE position is calculated every 40 ms.

Figure 1 - Block Diagram of Handover States

A DEVS model was created in order to simulate the Handover process on HomNets. The simulator is called HomNet simulator. The basic components are two main coupled models: MeNB and UE. MeNB stands for macro cell, and UE stands for User Equipment. The top level coupled model is the HomNet geographic area which is composed of a number of cells. Each cell contains one MeNB, and many UEs.

Each coupled model, MeNB and UE, is composed of two atomic models, a processor and a queue. The UE Queue is responsible for storing the messages received from the eNBs. A UE Queue has a ID. A UEProc has a ID, an initial position (currentX and currentY), a start time (sTime), an end time (etime), a final position (endX and endY), and a speed.

The UEProc is responsible for calculating the RSRP, the UE position, it is also responsible for requesting new messages stored on its Queue, processing the control messages received from the eNBs, and sending messages to the eNBs.

Queue stores messages from UEs and other eNBs. A Queue has a ID. MeNBProc is responsible for requesting new messages from the Queue, processing the messages and output other control messages to UEs and/or eNBs. A MeNBProc has a ID, a position (currentX and currentY), a frequency, and MeNBPower.

The eNBs are linked to each other through a X2 Link and they are linked to the UEs through a Radio Link.  Each MeNB has two outputs (Out and X2out) and two inputs (In and X2in).

The MeNB X2out port is connected to other MeNBs X2In port. The MeNB Out port is connected to UEs` In ports. The Req port is used to request new messages from the Queue.

Figure 2  - Structure of HomNet Simulator

Figure 3 - Message sequence for handover process

In the simulation scenarios, cells are considered to be macro cells in an urban area. A typical transmission power for a macro eNB is normally between 43 dBm to 48dBm. Therefore, the transmit power for a MeNB was set to 43dBm. The propagation model for macro cell is considered based on 3GPP standard in [1] and [2] as follows:

Macro Cell: 128.1 +37.6log10(d)

Figure 3 shows the simplified network architectures of simulation scenarios that are being used to analyze the handover performance based on different parameters suggested in 3GPP standard for LTE and LTE-Advanced cellular networks. The cell radius and antenna gain parameters were chosen to align with the specifications outlined in LTE release 12. As suggested in [2], [3], and [4], the frequency has been set to 2000 MHz for macro eNBs, the macro cell radius has been set to 500 m, and the antenna gain has been set to 12 dBi for the macro eNB, and 0 dBi for the UE.

Figure 4 -  Network Architecture

| ParametersValues |  |
| --- | --- |
| Number of macro eNBs | 7 |
| Number of UEs | 1, 50, 100 |
| UE Distribution | Uniform: randomly into the macro cell area |
| Frequency | 2000 MHz |
| Macro eNB Transmit power | 43 dBm |
| Cell Radius | 500 m |
| Antenna gain | 12 dBi (Macro eNB), and 0 dBi (UEs) |
| MCL | 70 dB |
| LogF | 10 dB |
| RSRP Sample | Every 40 ms |
| TTT (ms) | 160 |
| A3 offset | 3 dB |
| UE speed (km) | 30 |
| Handover preparation time | 50 ms |

Table 1  - Simulation assumptions

DEVS formal specification for each of the coupled and atomic models:

MHomNet = <X, Y, D, {Md |d∈D}, EIC, EOC, IC, select>

where,

X = Ø;

Y = Ø;

D=UEi,MeNBji,j∈ℕ

Md = {MUE,MMeNB} (where both MUE and MMeNB are coupled models);

EIC =  Ø

EOC =  Ø

[MeNBi,X2Out,MeNBj,X2Ini,j∈ℕ,i≠j, i.e. each MeNB does not connect to itself ];IC⊆[MeNBi,Out,UEk,Ini,k∈ℕ];[UEk,Out,MeNBi,Ini,k∈ℕ];

select = {UE, MeNB }.

MUE = <X, Y, D, {Md |d∈D}, EIC, EOC, IC, select>

where,

X = {(In,N)};

Y = {(Out,N)};

D = {Queue, UEProc}

Md = {MQueue ,MUEProc } (where both MQueue and MUEProc are atomic models);

EIC =  {((Self, In), (Queue, In)),};

EOC =  {((UEProc, Out), (Self, Out)) };

IC⊆[Queue,Out,UEProc,In];[UEProc,Req,Queue,Req]

select = {UEProc, Queue }.

MMeNB = <X, Y, D, {Md |d∈D}, EIC, EOC, IC, select>

where,

X = {(In, N),(X2In, N)};

Y = {(Out, N), (X2Out, N)};

D = {Queue, MeNBProc}

Md = {MQueue , MMeNBProc } (where both MQueue and MMeNBProc are atomic models);

EIC =  {((Self, In), (Queue, In)), ((Self, X2In), (Queue, X2In))};

EOC =  {((MeNBProc, Out), (Self, Out)), ((MeNBProc, X2Out), (Self, X2Out)) };

IC⊆[Queue,Out,UEProc,In];[UEProc,Req,Queue,Req]

select = {MeNBProc, Queue }.

Queue = <S, X, Y, δint, δext, λ, ta>

where,

S = {status∈Passive,Active, state∈Idle,Push,Pop,Send}; X = {In, Req}; Y = {Out};

δint(s) {

switch (state){

case Idle:

if (Request == 1) state = Send;

else passivate();

break;

case Push:

state = Idle;

break;

case Send:

Request = 0;

passivate();

break;

case Pop:

state = Idle;

break;

}

}

δext(s,e,x) {

if(x.port == In ||  x.port == X2in) {

if((x.destinationID == this->id || x.destinationID == 9999) && (x.sourceID != this->id){

add(x.value, elementsO);

state = Push;

}

else{ 	//message input port is Req

// 1 means add message to the queue.

//1-queue is empty

//2-queue is not empty

if (x.value == 1) {

if(sizeOf(elementsO) <= 0 ){

Request = 1;

passivate();

}

else {

Request = 1;

state = Send;

status = Active;

ta(status, ProcessTime);

}

}

else if(x.value == 2 && sizeOf(elementsO)>0){

// 2 means delete received message from the queue

//delete from the queue

state = Pop;

pop_front(elementsO);

Request = 0;

status = Active;

ta(status, ProcessTime);

}

}

}

}

λ(s) {

if ((state == Send) && (Request == 1)) {

sendOutput(msg.time(), Out, NULL, front(elementsO));

}

}

UEProc = <S, X, Y, δint, δext, λ, ta>

S = {status∈Passive,Active, state∈Idle,SendPack,RecPack}; X = {In}; Y = {Out, Req};

δint(s) {

switch (state){

case Idle:

state = SendPack;

break;

case RecPack:

state = Idle;

break;

case SendPack:

state = RecPack;

status = Active;

ta(status, ProcessTime);

break;

}

}

δext(s,e,x) {

if (x.port==In) {

state = SendPack;

if (x.message == CTRL_MeNB) {

//calculates the Serving eNB

}

else if (x.message == HO_CMD) {

//TeNB becomes SeNB i.e. concludes the HO process.

}

}

status = Active;

ta(status, ProcessTime);

}

λ(s) {

if (state == Idle){

sendOutput(msg.time(), Req, 1, NULL);

// 1 means UE processor requesting to queue to send him message

}

else if (state == SendPack) {

//calculates RSRP and the UE position.

//If HO criteria is met, it sends the RSRP to SeNB.

}

}

MeNBProc = <S, X, Y, δint, δext, λ, ta>

S = {status∈Passive,Active, state∈Idle,SendPack,RecPack}; X = {In};

Y = {X2Out,Out, Req};

δint(s) {

switch (state){

case Idle:

state = RecPack;

status = Active;

ta(status, ProcessTime);

break;

case RecPack:

state = Idle;

break;

case SendPack:

state = RecPack;

status = Active;

ta(status, ProcessTime);

break;

}

}

δext(s,e,x) {

if (x.port==In) {

// Received a new packet from Queue and will process and sent to others.

state = SendPack;

//For messages received from the UE

if (x.message == CSI_FEEDBACK) {

//Extract CSI Feedback.

//Set flags that define what kind of message will be sent.

}

//Received MR from the UE

else if (x.message == MES_REP) {

//Extract Measurement Report.

//Set flags that define what kind of message will be sent.

}

//Received HO_REQ from the  eNB

else if (x.message == HO_REQ) {

//Extract HO Request.

//Set flags that define what kind of message will be sent.

}

//Received HO_ACK from the  eNB

else if (x.message == HO_ACK) {

//Received HO Acknowledgement.

//Set flags that define what kind of message will be sent.

}

else if (x.message == CTRL_MeNB) {

//Received Control Command from other eNBs.

//extract the information received from the Control Command.

}

}

}

λ(s) {

if (ctrlCmdFlag == true){

// All the Macro eNBs share the system information and also send to UEs

CTRLcommand = new CTRLCmd(this->id, 9999, posx, posy, f, p);

sendOutput(msg.time(), Out, NULL, CTRLcommand);

sendOutput(msg.time(), X2out, NULL, CTRLcommand);

ctrlCmdFlag = false;

}

if (MRFlags[MRsourceUEid] == true){

// All the Macro eNBs share the system information and also send to UEs

//It sends HO Request

sendOutput(msg.time(), X2out, NULL, HOreq);

MRFlags[MRsourceUEid] = false;

}

if (HOReqFlags[HOReqSourceUEid] == true){

//It sends HO Request Acknowledgement

sendOutput(msg.time(), X2out, NULL, HOReqAcknowledgement);

HOReqFlags[HOReqSourceUEid] = false;

}

if (HOAckFlags[HOAcksourceUEid] == true){

//It sends RRC Command.

sendOutput(msg.time(), Out, NULL, RRCcmd);

HOAckFlags[HOAcksourceUEid] = false;

}

if (state == Idle){

sendOutput(msg.time(), Req, 1, NULL);

// 1 means UE processor requesting to queue to send him message

}

else if (state == SendPack) {

// It sends required messages to others (eNBs or UEs) and

//after sending the message it sends request to Queue to delete the received message

sendOutput(msg.time(), Req, 2, NULL);

}

}

Queue = <S, X, Y, δint, δext, λ, ta> (MeNB)

where,

S = {status∈Passive,Active, state∈Idle,Push,Pop,Send}; X = {X2In, In, Req};

Y = {Out};

δint(s) {

switch (state){

case Idle:

if (Request == 1) state = Send;

else passivate();

break;

case Push:

state = Idle;

break;

case Send:

Request = 0;

passivate();

break;

case Pop:

state = Idle;

break;

}

}

δext(s,e,x) {

if(x.port == In ||  x.port == X2in) {

if((x.destinationID == this->id || x.destinationID == 9999) && (x.sourceID != this->id){

add(x.value, elementsO);

state = Push;

}

else{ 	//message input port is Req

// 1 means add message to the queue.

//1-queue is empty

//2-queue is not empty

if (x.value == 1) {

if(sizeOf(elementsO) <= 0 ){

Request = 1;

passivate();

}

else {

Request = 1;

state = Send;

status = Active;

ta(status, ProcessTime);

}

}

else if(x.value == 2 && sizeOf(elementsO)>0){

// 2 means delete received message from the queue

//delete from the queue

state = Pop;

pop_front(elementsO);

Request = 0;

status = Active;

ta(status, ProcessTime);

}

}

}

}

λ(s) {

if ((state == Send) && (Request == 1)) {

sendOutput(msg.time(), Out, NULL, front(elementsO));

}

}

Developed software and testing strategy

This work was based on a previous work done in [5], therefore, a basic template for MeNBProc, UEProc, and Queue, was already ready for use. However, it could only calculate the received power of a UE without any movement or any kind of Handover procedure. Therefore, a class called movement.cpp was created in order to tackle the movement problem. A class previously created, called Msg.cpp, was modified and received new types of messages that assist on the HO procedure.

The HO algorithm described before was not implemented yet on MeNBProc and UEProc, then the classes UE.cpp, UE.h, MeNB.cpp, and MeNB.h were modified to meet the algorithm requirements. Also, the classes globalQueue.cpp and globalQueue.h were modified to meet the network architecture described before, by modifying the kind of ports used.

Since the basic templates were working well, it was not possible and even necessary to create an individual test strategy, therefore the development strategy was based on an incremental development with modification of individual classes but testing the execution of all the models at the same time.

The UE.cpp and UE.h were modified to use the class movement.cpp in order to calculate the UE`s position according to the simulation time. From the position, the received power can calculated. Thereafter, the HO procedure started to be tackled on the UEProc and MeNBProc sides. Since the HO procedure is based on the exchange of messages, the Msg.cpp and Msg.h were used to assist on the process. The classes UE.cpp and MeNB.cpp made use of Msg.cpp class.

In order to create the simulation scenarios, a script made in c++ was created to generate the ma files. It is called maFileGen. It generates ma files according to what is defined by the user inside maFileGen.cpp.

UML model

Figure 5 – UML model with the main components of the software.

Figure 6 – Extension of the UML model for the class Msg.

Simulation Results

The first test was made using only 1 UE, with speed of 30 km/h on a 7 MeNB architecture. It was made in order to ensure that the HO procedure was being done correctly. The UE starts at MeNB1 and goes to MeNB2. A part of the model.log file is showed as follows:

Mensaje * / 00:00:59:840 / Root(00) para top(01)

Mensaje * / 00:00:59:840 / top(01) para menb1(02)

Mensaje * / 00:00:59:840 / menb1(02) para menbp1(04)

Mensaje D / 00:00:59:840 / menbp1(04) / 00:00:00:000 para menb1(02)

Mensaje D / 00:00:59:840 / menb1(02) / 00:00:00:000 para top(01)

Mensaje D / 00:00:59:840 / top(01) / 00:00:00:000 para Root(00)

Mensaje * / 00:00:59:840 / Root(00) para top(01)

Mensaje * / 00:00:59:840 / top(01) para menb1(02)

Mensaje * / 00:00:59:840 / menb1(02) para menbp1(04)

Mensaje Y / 00:00:59:840 / menbp1(04) / req /      1.00000 para menb1(02)

Mensaje D / 00:00:59:840 / menbp1(04) / 00:00:00:001 para menb1(02)

Mensaje X / 00:00:59:840 / menb1(02) / req /      1.00000 para menbq1(03)

Mensaje D / 00:00:59:840 / menbq1(03) / ... para menb1(02)

Mensaje D / 00:00:59:840 / menb1(02) / 00:00:00:001 para top(01)

Mensaje D / 00:00:59:840 / top(01) / 00:00:00:000 para Root(00)

Mensaje * / 00:00:59:840 / Root(00) para top(01)

Mensaje * / 00:00:59:840 / top(01) para menb2(05)

Mensaje * / 00:00:59:840 / menb2(05) para menbp2(07)

Mensaje D / 00:00:59:840 / menbp2(07) / 00:00:00:000 para menb2(05)

Mensaje D / 00:00:59:840 / menb2(05) / 00:00:00:000 para top(01)

Mensaje D / 00:00:59:840 / top(01) / 00:00:00:000 para Root(00)

Mensaje * / 00:00:59:840 / Root(00) para top(01)

Mensaje * / 00:00:59:840 / top(01) para menb2(05)

Mensaje * / 00:00:59:840 / menb2(05) para menbp2(07)

Mensaje Y / 00:00:59:840 / menbp2(07) / req /      1.00000 para menb2(05)

Mensaje D / 00:00:59:840 / menbp2(07) / 00:00:00:001 para menb2(05)

Mensaje X / 00:00:59:840 / menb2(05) / req /      1.00000 para menbq2(06)

Mensaje D / 00:00:59:840 / menbq2(06) / ... para menb2(05)

Mensaje D / 00:00:59:840 / menb2(05) / 00:00:00:001 para top(01)

Mensaje D / 00:00:59:840 / top(01) / 00:00:00:000 para Root(00)

Mensaje * / 00:00:59:840 / Root(00) para top(01)

Mensaje * / 00:00:59:840 / top(01) para menb3(08)

Mensaje * / 00:00:59:840 / menb3(08) para menbp3(10)

Mensaje D / 00:00:59:840 / menbp3(10) / 00:00:00:000 para menb3(08)

Mensaje D / 00:00:59:840 / menb3(08) / 00:00:00:000 para top(01)

Mensaje D / 00:00:59:840 / top(01) / 00:00:00:000 para Root(00)

Mensaje * / 00:00:59:840 / Root(00) para top(01)

Mensaje * / 00:00:59:840 / top(01) para menb3(08)

Mensaje * / 00:00:59:840 / menb3(08) para menbp3(10)

Mensaje Y / 00:00:59:840 / menbp3(10) / req /      1.00000 para menb3(08)

Mensaje D / 00:00:59:840 / menbp3(10) / 00:00:00:001 para menb3(08)

Mensaje X / 00:00:59:840 / menb3(08) / req /      1.00000 para menbq3(09)

Mensaje D / 00:00:59:840 / menbq3(09) / ... para menb3(08)

Mensaje D / 00:00:59:840 / menb3(08) / 00:00:00:001 para top(01)

Mensaje D / 00:00:59:840 / top(01) / 00:00:00:000 para Root(00)

Mensaje * / 00:00:59:840 / Root(00) para top(01)

Mensaje * / 00:00:59:840 / top(01) para menb4(11)

Mensaje * / 00:00:59:840 / menb4(11) para menbp4(13)

Mensaje D / 00:00:59:840 / menbp4(13) / 00:00:00:000 para menb4(11)

Mensaje D / 00:00:59:840 / menb4(11) / 00:00:00:000 para top(01)

Mensaje D / 00:00:59:840 / top(01) / 00:00:00:000 para Root(00)

Mensaje * / 00:00:59:840 / Root(00) para top(01)

Mensaje * / 00:00:59:840 / top(01) para menb4(11)

Mensaje * / 00:00:59:840 / menb4(11) para menbp4(13)

Mensaje Y / 00:00:59:840 / menbp4(13) / req /      1.00000 para menb4(11)

Mensaje D / 00:00:59:840 / menbp4(13) / 00:00:00:001 para menb4(11)

Mensaje X / 00:00:59:840 / menb4(11) / req /      1.00000 para menbq4(12)

Mensaje D / 00:00:59:840 / menbq4(12) / ... para menb4(11)

Mensaje D / 00:00:59:840 / menb4(11) / 00:00:00:001 para top(01)

Mensaje D / 00:00:59:840 / top(01) / 00:00:00:000 para Root(00)

Mensaje * / 00:00:59:840 / Root(00) para top(01)

Mensaje * / 00:00:59:840 / top(01) para menb5(14)

Mensaje * / 00:00:59:840 / menb5(14) para menbp5(16)

Mensaje D / 00:00:59:840 / menbp5(16) / 00:00:00:000 para menb5(14)

Mensaje D / 00:00:59:840 / menb5(14) / 00:00:00:000 para top(01)

Mensaje D / 00:00:59:840 / top(01) / 00:00:00:000 para Root(00)

Mensaje * / 00:00:59:840 / Root(00) para top(01)

Mensaje * / 00:00:59:840 / top(01) para menb5(14)

Mensaje * / 00:00:59:840 / menb5(14) para menbp5(16)

Mensaje Y / 00:00:59:840 / menbp5(16) / req /      1.00000 para menb5(14)

Mensaje D / 00:00:59:840 / menbp5(16) / 00:00:00:001 para menb5(14)

Mensaje X / 00:00:59:840 / menb5(14) / req /      1.00000 para menbq5(15)

Mensaje D / 00:00:59:840 / menbq5(15) / ... para menb5(14)

Mensaje D / 00:00:59:840 / menb5(14) / 00:00:00:001 para top(01)

Mensaje D / 00:00:59:840 / top(01) / 00:00:00:000 para Root(00)

Mensaje * / 00:00:59:840 / Root(00) para top(01)

Mensaje * / 00:00:59:840 / top(01) para menb6(17)

Mensaje * / 00:00:59:840 / menb6(17) para menbp6(19)

Mensaje D / 00:00:59:840 / menbp6(19) / 00:00:00:000 para menb6(17)

Mensaje D / 00:00:59:840 / menb6(17) / 00:00:00:000 para top(01)

Mensaje D / 00:00:59:840 / top(01) / 00:00:00:000 para Root(00)

Mensaje * / 00:00:59:840 / Root(00) para top(01)

Mensaje * / 00:00:59:840 / top(01) para menb6(17)

Mensaje * / 00:00:59:840 / menb6(17) para menbp6(19)

Mensaje Y / 00:00:59:840 / menbp6(19) / req /      1.00000 para menb6(17)

Mensaje D / 00:00:59:840 / menbp6(19) / 00:00:00:001 para menb6(17)

Mensaje X / 00:00:59:840 / menb6(17) / req /      1.00000 para menbq6(18)

Mensaje D / 00:00:59:840 / menbq6(18) / ... para menb6(17)

Mensaje D / 00:00:59:840 / menb6(17) / 00:00:00:001 para top(01)

Mensaje D / 00:00:59:840 / top(01) / 00:00:00:000 para Root(00)

Mensaje * / 00:00:59:840 / Root(00) para top(01)

Mensaje * / 00:00:59:840 / top(01) para menb7(20)

Mensaje * / 00:00:59:840 / menb7(20) para menbp7(22)

Mensaje D / 00:00:59:840 / menbp7(22) / 00:00:00:000 para menb7(20)

Mensaje D / 00:00:59:840 / menb7(20) / 00:00:00:000 para top(01)

Mensaje D / 00:00:59:840 / top(01) / 00:00:00:000 para Root(00)

Mensaje * / 00:00:59:840 / Root(00) para top(01)

Mensaje * / 00:00:59:840 / top(01) para menb7(20)

Mensaje * / 00:00:59:840 / menb7(20) para menbp7(22)

Mensaje Y / 00:00:59:840 / menbp7(22) / req /      1.00000 para menb7(20)

Mensaje D / 00:00:59:840 / menbp7(22) / 00:00:00:001 para menb7(20)

Mensaje X / 00:00:59:840 / menb7(20) / req /      1.00000 para menbq7(21)

Mensaje D / 00:00:59:840 / menbq7(21) / ... para menb7(20)

Mensaje D / 00:00:59:840 / menb7(20) / 00:00:00:001 para top(01)

Mensaje D / 00:00:59:840 / top(01) / 00:00:00:000 para Root(00)

Mensaje * / 00:00:59:840 / Root(00) para top(01)

Mensaje * / 00:00:59:840 / top(01) para ue3591(23)

Mensaje * / 00:00:59:840 / ue3591(23) para uep3591(25)

Mensaje D / 00:00:59:840 / uep3591(25) / 00:00:00:000 para ue3591(23)

Mensaje D / 00:00:59:840 / ue3591(23) / 00:00:00:000 para top(01)

Mensaje D / 00:00:59:840 / top(01) / 00:00:00:000 para Root(00)

Mensaje * / 00:00:59:840 / Root(00) para top(01)

Mensaje * / 00:00:59:840 / top(01) para ue3591(23)

Mensaje * / 00:00:59:840 / ue3591(23) para uep3591(25)

Mensaje Y / 00:00:59:840 / uep3591(25) / req /      1.00000 para ue3591(23)

Mensaje D / 00:00:59:840 / uep3591(25) / 00:00:00:000 para ue3591(23)

Mensaje X / 00:00:59:840 / ue3591(23) / req /      1.00000 para ueq3591(24)

Mensaje D / 00:00:59:840 / ueq3591(24) / ... para ue3591(23)

Mensaje D / 00:00:59:840 / ue3591(23) / 00:00:00:000 para top(01)

Mensaje D / 00:00:59:840 / top(01) / 00:00:00:000 para Root(00)

Mensaje * / 00:00:59:840 / Root(00) para top(01)

Mensaje * / 00:00:59:840 / top(01) para ue3591(23)

Mensaje * / 00:00:59:840 / ue3591(23) para uep3591(25)

Mensaje Y / 00:00:59:840 / uep3591(25) / out /      0.00000 para ue3591(23)

Mensaje Y / 00:00:59:840 / uep3591(25) / req /      2.00000 para ue3591(23)

Mensaje D / 00:00:59:840 / uep3591(25) / 00:00:00:001 para ue3591(23)

Mensaje Y / 00:00:59:840 / ue3591(23) / out /      0.00000 para top(01)

Mensaje X / 00:00:59:840 / ue3591(23) / req /      2.00000 para ueq3591(24)

Mensaje X / 00:00:59:840 / top(01) / in /      0.00000 para menb1(02)

Mensaje X / 00:00:59:840 / top(01) / in /      0.00000 para menb2(05)

Mensaje X / 00:00:59:840 / top(01) / in /      0.00000 para menb3(08)

Mensaje X / 00:00:59:840 / top(01) / in /      0.00000 para menb4(11)

Mensaje X / 00:00:59:840 / top(01) / in /      0.00000 para menb5(14)

Mensaje X / 00:00:59:840 / top(01) / in /      0.00000 para menb6(17)

Mensaje X / 00:00:59:840 / top(01) / in /      0.00000 para menb7(20)

Mensaje D / 00:00:59:840 / ueq3591(24) / ... para ue3591(23)

Mensaje X / 00:00:59:840 / menb1(02) / in /      0.00000 para menbq1(03)

Mensaje X / 00:00:59:840 / menb2(05) / in /      0.00000 para menbq2(06)

Mensaje X / 00:00:59:840 / menb3(08) / in /      0.00000 para menbq3(09)

Mensaje X / 00:00:59:840 / menb4(11) / in /      0.00000 para menbq4(12)

Mensaje X / 00:00:59:840 / menb5(14) / in /      0.00000 para menbq5(15)

Mensaje X / 00:00:59:840 / menb6(17) / in /      0.00000 para menbq6(18)

Mensaje X / 00:00:59:840 / menb7(20) / in /      0.00000 para menbq7(21)

Mensaje D / 00:00:59:840 / ue3591(23) / 00:00:00:001 para top(01)

Mensaje D / 00:00:59:840 / menbq1(03) / ... para menb1(02)

Mensaje D / 00:00:59:840 / menbq2(06) / ... para menb2(05)

Mensaje D / 00:00:59:840 / menbq3(09) / ... para menb3(08)

Mensaje D / 00:00:59:840 / menbq4(12) / ... para menb4(11)

Mensaje D / 00:00:59:840 / menbq5(15) / ... para menb5(14)

Mensaje D / 00:00:59:840 / menbq6(18) / ... para menb6(17)

Mensaje D / 00:00:59:840 / menbq7(21) / ... para menb7(20)

Mensaje D / 00:00:59:840 / menb1(02) / 00:00:00:001 para top(01)

Mensaje D / 00:00:59:840 / menb2(05) / 00:00:00:001 para top(01)

Mensaje D / 00:00:59:840 / menb3(08) / 00:00:00:001 para top(01)

Mensaje D / 00:00:59:840 / menb4(11) / 00:00:00:001 para top(01)

Mensaje D / 00:00:59:840 / menb5(14) / 00:00:00:001 para top(01)

Mensaje D / 00:00:59:840 / menb6(17) / 00:00:00:001 para top(01)

Mensaje D / 00:00:59:840 / menb7(20) / 00:00:00:001 para top(01)

Mensaje D / 00:00:59:840 / top(01) / 00:00:00:001 para Root(00)

The previous result was taken from the time where the HO procedure happened. It is possible to visualize that MeNB1 is communicating with MeNB2 and UE1 in order to do the HO procedure. It is also worth noting that the previous result is really long, even though it only shows the logs for one time stamp. Therefore, the other simulation runs were not ran with model.log to decrease the simulation time. Moreover, for the sake of the analysis, model.log is not really important, since what is needed is the amount of HO that occurred. The file resultCount.txt shows that information. The files logfile.txt and receivedMsg.txt were created to ensure the correctness of the results since they show the information concerning received power, UE position, etc. The follow text shows a bit of those files from the result of the first simulation scenario.

From logfile.txt:

@UE: 3591 eNB ID: 1 RECEIVED THE CSI_FEEDBACK FROM  UE ID: 3591

@UE: 3591 Verifying HO  msg.time().asMsecs() 59680 TRAVELED DISTANCE: 497.333 metres at position: (1893, 1893) Total distance to be done: 707.107 metres  ActualServingID: 1 ActualServingPower: -71 NewServingID: 2 NewServingPower: -68

@UE: 3591 Verifying HO  msg.time().asMsecs() 59720 TRAVELED DISTANCE: 497.667 metres at position: (1893, 1893) Total distance to be done: 707.107 metres  ActualServingID: 1 ActualServingPower: -71 NewServingID: 2 NewServingPower: -68

@UE: 3591 Verifying HO  msg.time().asMsecs() 59760 TRAVELED DISTANCE: 498 metres at position: (1893, 1893) Total distance to be done: 707.107 metres  ActualServingID: 1 ActualServingPower: -71 NewServingID: 2 NewServingPower: -68

@UE: 3591 Verifying HO  msg.time().asMsecs() 59800 TRAVELED DISTANCE: 498.333 metres at position: (1893, 1893) Total distance to be done: 707.107 metres  ActualServingID: 1 ActualServingPower: -71 NewServingID: 2 NewServingPower: -68

@UE: 3591 Verifying HO  msg.time().asMsecs() 59840 TRAVELED DISTANCE: 498.667 metres at position: (1894, 1894) Total distance to be done: 707.107 metres  ActualServingID: 1 ActualServingPower: -71 NewServingID: 2 NewServingPower: -68

@UE: 3591 TTT  msg.time().asMsecs() 59840 TRAVELED DISTANCE: 498.667 metres at position: (1894, 1894) Total distance to be done: 707.107 metres  ActualServingID: 1 ActualServingPower: -71 NewServingID: 2 NewServingPower: -68

@UE: 3591 eNB ID: 1 RECEIVED THE MR FROM  UE ID: 3591 MR->getTargetID() 2

MRtargedID 2

@UE: 3591 eNB ID: 2 RECEIVED THE HO_REQ FROM  eNB ID: 1 HOReqtargedID 2

@UE: 3591 eNB ID: 1 RECEIVED THE HO_ACK FROM  eNB ID: 2 HOAcktargedID 2

UE ID: 3591 RECEIVED THE HO_CMD FROM  eNB ID: 1 TeNB 2

UE: 3591 reached final Position 707 metres

From logfile.txt:

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%@UE 3591 CSIFeedback time13

The results for the other scenarios are in the folder results, of the simulator. The number of HO for the scenario in the file 50UEs7MeNBVel30HomNet.ma which has 50 UEs was 68, for the scenario in the file 100UEs7MeNBVel30HomNet.ma which has 100 UEs was 130.

References

[1]  “3GPP TS 36.814”.

[2]  “3GPP TR 36.842”.

[3]  3GPP, "TR 36.932”.

[4]  3GPP, "TS 23.0009".

[5]  B. U. Kazi, M. Etemad, G. Wainer, R. Casselman, and G. Boudreau, "Using Elected Coordination Station for Heterogeneous Multicell Cooperative Networks ". 2016